01  What Is LiDAR?

Understanding the core sensing technology behind this project

DEFINITION

LiDAR stands for Light Detection and Ranging. It works by emitting laser pulses and measuring the time it takes for each pulse to bounce back from a surface. That travel time is converted into a precise distance measurement — the same principle as sonar, but using light instead of sound.

HOW A DISTANCE READING BECOMES A 3D POINT

The sensor fires a laser and measures the round-trip time to the target.

Distance (d) is calculated:  d = (c × t) / 2

An orientation reading (from the IMU) provides a direction vector.

Together they place a point in 3D space: (x, y, z)

Thousands of these points build a point cloud — a 3D map.

REAL-WORLD USES

Self-driving cars

Robotics & navigation

Architecture & surveying

Forestry & geology

AR/VR spatial computing

Archaeology

▶  This project makes LiDAR accessible at low cost — from a $30 sensor to a live interactive 3D point cloud.

KEY STATS

Sample rate: 250 Hz

Effective range: 0.2 – 8 m

Typical accuracy: ±2 cm

Output: 3D point cloud

02  Hardware Components

Every physical part of the scanner and what it contributes

TF-Luna LiDAR  |  Measures distance via time-of-flight laser   [UART (serial)]   250 Hz, 0.2–8 m; returns dist, signal strength & temp

BNO055 IMU  |  Provides 9-DOF orientation as a quaternion for point direction   [I²C (bus 1)]   On-board sensor fusion; outputs qw, qx, qy, qz directly

Raspberry Pi 5  |  Central compute — reads sensors, processes data, sends UDP   [GPIO / USB]   Runs Python tools, hosts the HAL, streams to visualization

Pi Camera Rev 1.3  |  Detects ArUco visual markers for precise scanner positioning   [CSI ribbon]   Used with OpenCV for pose estimation; replaced Pi Camera 3

3D Printed Housing  |  Holds all electronics in a stable handheld form factor   [—]   Designed in Rhino 3D; maintains sensor alignment during scans

ArUco Markers  |  Visual reference points to anchor the scanner in space   [Camera]   Reduce IMU drift; provide absolute position in the scene

▶  Each component was validated with isolated Python test scripts before full integration — catching wiring and config issues early.

TOTAL COMPONENT COST

TF-Luna (~$30), BNO055 breakout (~$10), Raspberry Pi 5 (~$80), Pi Camera (~$25). The system proves that capable spatial scanning does not require expensive commercial hardware.

03  Physical Construction

Wiring, CAD design, 3D printing, and final assembly

WIRING THE SCANNER

TF-Luna → Pi UART: TX→RX (GPIO 14/15), 5 V power & GND

BNO055 → Pi I²C: SDA (GPIO 2), SCL (GPIO 3), 3.3 V & GND

Pi Camera: CSI ribbon connector to Pi 5 camera port

UART and I²C enabled in raspi-config; serial console disabled

User added to dialout group for serial port access

CAD & 3D PRINTING

Housing modeled in Rhino 3D (.3dm files)

Designed to hold TF-Luna, BNO055, Pi 5, and camera together

Sensor alignment baked into the geometry — no re-calibration needed after assembly

Printed while software development ran in parallel to save time

ASSEMBLY STEPS

Step 1: Wire TF-Luna to Pi and validate with tfluna_read.py

Step 2: Wire BNO055 to Pi and validate with bno055_quat_read.py

Step 3: Connect Pi Camera and verify image capture

Step 4: Run camera calibration and capture ArUco marker images

Step 5: Install components into 3D printed housing

Step 6: Re-run all sensor tests as an integrated unit

▶  The housing was designed so the LiDAR sensor, IMU, and camera maintain a fixed spatial relationship — making orientation math between sensors consistent.

04  Software Architecture

Python tools, HAL interfaces, and the visualization stack

PYTHON SIDE (RASPBERRY PI)

HAL (Hardware Abstraction Layer) — clean interfaces; any sensor can be swapped or mocked without changing other code

tfluna_read.py — UART reader for raw distance frames

bno055_quat_read.py — I²C reader for quaternion orientation

aruco_pose_demo.py — camera-based marker detection and pose estimation

send_sensor_data_udp.py — fuses all sensor streams and broadcasts UDP packets

TESTING & MOCKING

All HAL interfaces have mock implementations for offline testing

tests/test_hal.py verifies sensor logic without hardware

Mock output logs in test_outputs/ used for data format validation

VISUALIZATION SIDE (UNITY)

Unity (C#) — rendering target; validated the full data pipeline

Custom GLSL point cloud shader for real-time rendering

Debug overlay for live diagnostics during scanning

▶  The HAL design means the scanner's software can be tested on any computer — not just the Pi — which dramatically speeds up development.

DATA FLOW

TF-Luna (UART) ──→ Pi Python tools ──→ Coordinate transform ──→ UDP packet ──→ Unity receiver ──→ Point Cloud render
BNO055 (I²C) ──→ ↗                  ArUco Camera ──→ ↗ (scanner origin fusion)

05  Initial Testing

Validating every component before full system integration

TF-LUNA RAW OUTPUT SAMPLE

1770662039.648  dist_cm=0   strength=128  temp_c=39.00
1770662039.658  dist_cm=0   strength=128  temp_c=39.00
1770662039.669  dist_cm=42  strength=131  temp_c=39.00

Each frame includes a Unix timestamp, measured distance, return signal strength (quality), and on-chip temperature.

BNO055 RAW OUTPUT SAMPLE

1770662963.567  qw=1.000 qx=0.000 qy=-0.000 qz=0.000
1770662963.669  qw=0.368 qx=-0.492 qy=0.788 qz=0.000
1770662963.771  qw=0.368 qx=-0.492 qy=0.788 qz=0.000

Quaternions describe full 3D orientation with no gimbal lock. The BNO055's internal fusion removes the need for manual Euler math.

INCREMENTAL TEST STRATEGY

Each sensor tested in isolation before integration

Hardware faults caught early via per-component scripts

UDP stream validated before connecting to Unity

ArUco pipeline tested independently from IMU stream

Camera calibration verified with dedicated image capture tool

▶  Testing components separately first eliminated wiring and config issues early, so final integration debugging focused only on software logic.

06  Sending Data with UDP

Bridging the Raspberry Pi scanner to the visualization environment in real time

WHY UDP?

Low overhead — no handshaking or connection state

Low latency — packets arrive as fast as the network allows

Fire-and-forget — acceptable if a few frames are dropped; point clouds are naturally tolerant of gaps

Easy to receive — Unity and Python both have simple UDP socket APIs

PACKET CONTENTS

tfluna — distance (cm), signal strength, temperature

bno055 — quaternion orientation (qw, qx, qy, qz)

aruco — detected marker IDs and pose vectors (when camera is active)

scanner_pose — fused position (from ArUco) + orientation (from IMU)

pos_m — final 3D endpoint in world space, ready to plot

TRANSMISSION PIPELINE

Pi reads TF-Luna and BNO055 continuously

ArUco detector sends marker poses to local port 6006

Aggregator fuses all three streams into one JSON packet

Packet broadcast to visualization host on port 5005

Unity receiver parses JSON and queues the new point

▶  The Pi owns the coordinate math. The visualization engine just plots what arrives — keeping the rendering side simple and engine-agnostic.

FRAME MODE TAG

Each packet includes a frame mode tag (unity_frame / raw) so the receiving engine knows whether coordinate conversion has already been applied by the Pi — eliminating double-correction bugs.

07  Point Cloud Generation

Turning raw sensor samples into a navigable 3D spatial map

HOW EACH POINT IS COMPUTED

1. Read distance — TF-Luna returns distance d to the nearest surface along the sensor axis.

2. Read orientation — BNO055 returns a unit quaternion q describing where the scanner is pointing.

3. Build direction vector — rotate the scanner's forward vector (0,0,1) by q to get a world-space direction.

4. Compute endpoint — multiply the direction vector by d to get the absolute 3D position of the surface.

5. Accumulate — the new point is appended to the cloud buffer and rendered in real time.

COORDINATE SYSTEM CHALLENGES

TF-Luna, BNO055, and Unity each use different axis conventions

Iterative live testing used to determine axis flips, swaps, and yaw offsets

Final calibrated settings: Flip Y, Swap Y↔Z, Invert Z, Yaw correction −180°

Frame mode tagging prevents double-correction between Pi and engine

RENDERING

Custom GLSL point cloud shader

Points colored by depth or height

GPU-side rendering for performance

Renders in Unity in real time

▶  ArUco markers improve accuracy by anchoring the scanner's 3D position. Without them, small IMU drift causes scans to 'float' over time.

08  Conclusion & Lessons Learned

What was built, what was gained, and where it goes next

WHAT WAS DELIVERED

Fully wired handheld LiDAR scanner in a custom 3D printed housing

Python sensor stack with HAL interfaces and hardware mock support

ArUco camera pipeline for accurate scanner pose estimation

UDP sensor fusion stream combining LiDAR, IMU, and camera data

Unity point cloud viewer — validated end-to-end pipeline

KEY LESSONS

Test incrementally — isolate components first, integrate second

Pivot strategically — when a camera driver blocked progress, shifting focus to Unity visualization kept the project moving

Coordinate systems matter — every engine and sensor has its own axes; mapping them correctly is non-trivial

Sensor fusion > single-source — combining IMU + ArUco produces far more stable results than either alone

SKILLS DEVELOPED

Embedded hardware integration (UART, I²C, GPIO)

Computer vision (camera calibration, ArUco detection)

3D math: quaternions, coordinate transforms, point clouds

Network programming (UDP socket design)

Game engine development (Unity C#)

CAD design and 3D printing

▶  This project combined hardware, software, networking, computer vision, and 3D math into a single working pipeline — demonstrating what a computer science education looks like in practice.

FUTURE DIRECTIONS

SLAM — simultaneous localization and mapping during movement

Point cloud export and mesh reconstruction

Smoothed sensor fusion with complementary or Kalman filtering